Step-by-step: flushing your lines

1. Start with cold water only

2. Open the lowest faucet first

This is usually:

- Basement laundry sink

- Garage hose bib

- First-floor tub

Open it fully cold and let it run 5–10 minutes until the water is clear and steady.

3. Work your way up

Go floor by floor:

- Open cold water at sinks, tubs, showers

- Let each run 2–3 minutes

- Toilets: flush each one 2–3 times

4. Flush appliances

After water runs clear:

- Run washing machine on a cold rinse

- Run dishwasher without dishes

- If you have a fridge water line, dispense 2–3 gallons

5. Now flush the hot water system

Once cold water is clean:

- Turn on hot taps one at a time

- Let them run 2–3 minutes